How to install Ubuntu and ROS2 in Jetson Nano

- Install xubuntu in Jetson Nano

In this Repository I will show how to install Ubuntu & ROS2 on a Jetson Nano

We will download (Xubuntu 20.04 Focal Fossa L4T R32.3.1) instead of Ubuntu because it is a custom image for the Jetson Nano

- Open ( https://forums.developer.nvidia.com/t/xubuntu-20-04-focal-fossa-l4t-r32-3-1-custom-image-for-the-jetson-nano/121768)

- Click on the download link : Xubuntu-20.04-l4t-32.3.1.tar.tbz2

- After the following window pops up, click on Save file>OK

- Download balena

- You need to flash your SD card using balenaEtcher (Recommended) , here is the link that will do that for you: https://www.balena.io/etcher/ , click Download for Linux x64 > Save File> OK

- Click on Extract Here so you can have the app image

- Now  the program is set up

- Before you can flash the image you have to Extract the zip file for ubuntu that you’ve download it using the Terminal type this command to extract the Xubuntu image:

tar -xvjf Xubuntu-20.04-l4t-r32.3.1.tar.tbz2

- Go back to the balena Etcher > Flash from file> Downloads>Xubuntu-20.04…. (Select the image for xubuntu)

- Plug in the USB drive.

- Identify the USB drive or click Change to select a different USB.

- Select Target and select your device then click on Flash

Then you will have to put in your password for the authentication

- The Flashing will start. This will take some time, you will have to wait until the flash is completed

- When the process completes, close BalenaEtcher.

- If your Jetson Nano is A02 then you have to plug in the SD card, and after its booting, it will take you to the XUbuntu installation screen.

- To find out which .dtb file is compatible with your Jetson Nano run the following command on its terminal : cat /sys/firmware/devicetree/base/compatible

If your Jetson Nano was B01 version,  from your SD card go to boot > extlinux > extlinux.conf then open it in terminal and enter the following command: sudo vim extlinux.conf

then add this line in the extlinux.conf and save the file: FDT /boot/tegra210-p3448-0000-p3449-0000-b00.dtb

Finally we plug in the SD card, and after booting, it will take you to the XUbuntu installation screen.

- ROS2 in Jetson Nano

- Installing ROS2 on Jetson Nano is pretty straightforward, all you have to do is to follow the instructions provided in this link   https://docs.ros.org/en/foxy/Installation/Ubuntu-Install-Debians.html